CPE301 – Fall 2019

# Design Assignment 2C

Student Name: Jacob Patrick Reed

Student #: 1008448895

Student Email: reedj35@unlv.nevada.edu

Primary Github address: https://github.com/reedjacobp

Directory: https://github.com/reedjacobp/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ATmega328PB Xplained Mini

Multi-function shield

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

/*

* DA2C_Task1.c

*

* Created: 10/9/2019 10:56:08 PM

* Author : jreed

*/

#define F_CPU 16000000UL

#include <avr/io.h>

int main(void)

{

DDRB = 0xFF; //all registers in DDRB are outputs

DDRC = 0x00; //all registers in DDRC are inputs

PORTB = 0xFF; //turn on all LEDs

TCCR0A = 0x00; //normal operation

TCNT0 = 0x00; //start the timer

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescaler to 1024

int count = 0; //use this to count overflow

//The fout for this is about 61Hz

while(1)

{

count = 0;

TCNT0 = 0x00; //start the timer

while(count < 23) //60% off

{

if(TCNT0 == 0xFF) //check to see if TOP(255) has been reached

{

count++; //upon overflow, count it, then reset timer

TCNT0 = 0x00; //reset timer

}

else if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB |= (1 << 3); //turn off LED @ PB3

PORTB &= ~(1 << 2); //turn on LED @ PB2

TCNT0 = 0x00; //reset timer if switch is pushed

count = 0;

while(count < 82) //1.333 seconds

{

if(TCNT0 == 0xFF)

{

count++;

TCNT0 = 0x00;

}

}

PORTB |= (1 << 2); //turn off LED @ PB2

}

}

PORTB ^= (1 << 3); //once overflow has been counted 23 times, turn off LED by XOR

count = 0; //reset the count

TCNT0 = 0x00; //since timer is constantly incrementing, I must reset right before a loop uses it

while(count < 15) //40% on

{

if(TCNT0 == 0xFF) //check to see if TOP(255) has been reached

{

count++; //upon overflow, count it, then reset timer

TCNT0 = 0x00; //reset timer

}

else if(!(PINC & (1 << 3))) //check if SW3 is pushed

{

PORTB |= (1 << 3);

PORTB &= ~(1 << 2);

count = 0;

TCNT0 = 0x00; //reset timer if switch is pushed

while(count < 82) //1.333 seconds

{

if(TCNT0 == 0xFF)

{

count++;

TCNT0 = 0x00;

}

}

PORTB |= (1 << 2);

}

}

PORTB ^= (1 << 3); //once overflow has been counted 12 times, turn on LED by XOR

}

return 0;

}

- DEVELOPED MODIFIED CODE OF TASK 2

/*

* DA2C_Task2.c

*

* Created: 10/10/2019 10:33:17 PM

* Author : jreed

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

uint8_t OVF_COUNT = 0;

uint8_t OVF_LIMIT = 16;

int main(void)

{

//int count = 0;

DDRB = 0xFF; //all registers in DDRB are outputs

DDRC = 0x00; //all registers in DDRC are inputs

PORTB = 0xFF; //turn on all LEDs

TCCR0A = 0x00; //normal operation

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescalar to 1024

TCNT0 = 12;

TIMSK0 |= (1 << TOIE0);

sei();

while (1)

{

}

}

ISR (TIMER0_OVF_vect)

{

OVF_COUNT++;

if (OVF_COUNT == OVF_LIMIT)

{

PORTB ^= (1 << 3);

PORTB |= (1 << 2);

if(OVF_LIMIT == 16)

{

OVF_LIMIT = 24;

}

if(!(PINC & (1 << 3)))

{

OVF_LIMIT = 85;

PORTB |= (1 << 3);

PORTB &= ~(1 << 2);

}

else

{

OVF_LIMIT = 16;

}

OVF_COUNT = 0;

}

TCNT0 = 12;

}

- DEVELOPED MODIFIED CODE OF TASK 3

/*

* DA2C_Task3.c

*

* Created: 10/11/2019 12:39:36 AM

* Author : jreed

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

uint8_t OVF_COUNT = 0;

uint8_t OVF_LIMIT = 16;

int main(void)

{

DDRB = 0xFF; //all registers in DDRB are outputs

DDRC = 0x00; //all registers in DDRC are inputs

PORTB = 0xFF; //turn on all LEDs

OCR0A = 244; //normal operation

TCNT0 = 0;

TCCR0A |= (1 << WGM01) | (1 << COM1A0);

TCCR0B |= (1 << CS02) | (1 << CS00); //set prescalar to 1024

TIMSK0 |= (1 << OCIE0A);

sei();

while (1)

{

}

}

ISR (TIMER0_COMPA_vect)

{

OVF_COUNT++;

if (OVF_COUNT == OVF_LIMIT)

{

PORTB ^= (1 << 3);

PORTB |= (1 << 2);

if(OVF_LIMIT == 16)

{

OVF_LIMIT = 24;

if(!(PINC & (1 << 3)))

{

OVF_LIMIT = 85;

PORTB |= (1 << 3);

PORTB &= ~(1 << 2);

}

}

else

{

OVF_LIMIT = 16;

if(!(PINC & (1 << 3)))

{

OVF_LIMIT = 85;

PORTB |= (1 << 3);

PORTB &= ~(1 << 2);

}

}

OVF_COUNT = 0;

}

TCNT0 = 0;

}

- SCHEMATICS

N/A

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1 Waveform Screenshot

Task 2 Waveform Screenshot

Task 3 Waveform Screenshot

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

Task 1 : https://youtu.be/oHMrpV3Dsxo

Task 2 : https://youtu.be/qoYsYyKb3F0

Task 3 : https://youtu.be/V4uJRaN4pp0

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Jacob Reed