CPE301 – Fall 2019

# Design Assignment 2B

Student Name: Jacob Patrick Reed

Student #: 1008448895

Student Email: reedj35@unlv.nevada.edu

Primary Github address: https://github.com/reedjacobp

Directory: https://github.com/reedjacobp/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA3B.c

*

* Created: 10/20/2019 8:48:28 PM

* Author : jreed

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void USART_init(void); //function to initialize USART

unsigned char USART_receive(void); //function to receive through USART

void USART_send(unsigned char data); //function to send through USART

void USART_putstring(char* StringPtr); //function to scan through the string

float temp;

uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

int main(void)

{

USART_init(); //initialize USART

ADCSRA = 0x84;

ADMUX = 0x44;

TCCR0A = 0x00; //normal operation

TCCR0B |= (1 << CS02); //set prescalar to 256

TCNT0 = 16; //TOP = 256-250 = 16

TIMSK0 |= (1 << TOIE0);

sei(); //enable interrupt

while (1)

{

}

}

void USART_init(void)

{

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

unsigned char USART_receive(void)

{

while(!(UCSR0A & (1 << RXC0)));

return UDR0;

}

void USART_send(unsigned char data)

{

while(!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_putstring(char* StringPtr)

{

while(*StringPtr != 0x00)

{

USART_send(*StringPtr);

StringPtr++;

}

}

ISR (TIMER0_OVF_vect)

{

OVF_COUNT++; //increment the overflow counter

if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

{

ADCSRA |= (1 << ADSC);

while((ADCSRA & (1 << ADIF)) == 0)

{

ADCSRA |= (1 << ADIF);

int temp = ADCL;

temp = temp | (ADCH<<8);

temp = (temp/1024.0) * 5000/10;

temp = (temp*2)+32; //equation to convert celsius to farenheit. can't use used 2 instead of 9/5

//USART_send((a/100)+'0');

temp = temp % 100;

USART_send((temp/10)+'0');

temp = temp % 10;

USART_send((temp)+'0');

//USART_send('\r');

USART_send('\n');

}

OVF_COUNT = 0; //reset overflow counter

}

TCNT0 = 16; //reset TOP

- }DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

N/A

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/SWATbKt9W2s

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT