CPE301 – Fall 2019

# Design Assignment 4A

Student Name: Jacob Patrick Reed

Student #: 1008448895

Student Email: reedj35@unlv.nevada.edu

Primary Github address: https://github.com/reedjacobp

Directory: https://github.com/reedjacobp/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- ATmega328PB Xplained Mini

- Multifunction Shield

- TB6612FNG Motor Driver

- DC Motor

- Logic Analyzer

- DEVELOPED CODE OF TASK 1

/*

* DA4A.c

*

* Created: 10/31/2019 4:26:26 PM

* Author : jreed

*/

#define F_CPU 16000000UL	// 16MHz

#include <avr/io.h>		    //

#include <avr/interrupt.h>  //

#include <util/delay.h>		//

void init_adc(void);

int control;	//  VARIABLE FOR ADC VALUE (NOT NEEDED)

int toggle = 0; // USED TO DETERMINE IF BUTTON SHOULD BE ON OR OFF

int main()

{

DDRB |= (1 << DDRB1) | (1 << DDRB2); //make PB1 and PB2 as output

DDRD |= (1 << DDRD6); //make PD6 as output for PWM

DDRC &= ~(1 << DDRC0); //PC0 is an input

DDRC &= ~(1 << DDRC1); //PC1 is an input

PORTC |= (1 << PORTC1);	// ENABLE PULL-UP RESISTOR

TCCR0A=0x83; //set up for fast PWM and clear OC0A on compare match

TCCR0B=0x05; //prescalar of 1024

PCICR = 0x02; //enable PCIE1 for PCMSK1 to work

PCMSK1 = 0x02; //enable pin changes on PC1 (PCINT9)

sei(); //enable interrupts

ADMUX = (1<<REFS0); //reference voltage at AREF

ADCSRA = (1<<ADEN)|(1<<ADSC)|(1<<ADATE)|(1<<ADPS2)|(1<<ADPS1)|(1<<ADPS0); //set up for ADC conversion

while (1)

{

//wait here

}

}

ISR(PCINT1_vect){

if(!(PINC & (1<<PINC1))){

if(toggle == 0){

OCR0A = 0;

PORTB &= ~(1<<PORTB2);

_delay_ms(1000);

}

if (toggle == 1){

while((ADCSRA&(1<<ADIF))==0); // wait for conversion

control = ADC;				  // ADC Conversion

OCR0A = control;			  // Output to converted value to 0CR0A

PORTB |= (1 << PORTB2);

_delay_ms(1000);

}

toggle ^= 1; //update state of motor to on

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

With the potentiometer decreased to minimum

With the potentiometer increase to maximum

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/_vvTdJz7MJ8

- GITHUB LINK OF THIS DA

https://github.com/reedjacobp/submission_da/tree/master/DesignAssignments/DA4A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT