CPE301 – Fall 2019

# Final Project

Student Name: Jacob Patrick Reed

Student #: 1008448895

Student Email: reedj35@unlv.nevada.edu

Primary Github address: https://github.com/reedjacobp

Directory: https://github.com/reedjacobp/submission_da

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

ATmega328PB Xplained Mini

ESP Module

APDS9960

- DEVELOPED CODE OF TASK 1

/*

* FinalProject.c

*

* Created: 12/8/2019 12:32:56 AM

* Author : jreed

*/

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

// Global constants for uart

#define BAUD 115200

#define FOSC 16000000

#define UBRR FOSC/8/BAUD-1

#define APDS9960_WRITE 0x72

#define APDS9960_READ 0x73

//include standard libraries

#include <avr/io.h>

#include <stdlib.h>

#include <stdio.h>

#include <util/delay.h>

#include <math.h>

//include custom libraries

#include "APDS9960_def.h"

#include "i2c_master.h"

//Function declarations

void getValues(void);

// void TIMER1_init();

void init_APDS9960(void);

void usart_init();

void USART_putstring(volatile unsigned char *StringPtr);

//AT commands

volatile unsigned char AT[] = "AT\r\n"; // Test

volatile unsigned char CWMODE[] = "AT+CWMODE=3\r\n"; // Set Wi-Fi mode

volatile unsigned char CWJAP[] = "AT+CWJAP=\"blahblah\",\"blahblah\"\r\n"; // Get Wi-Fi info

volatile unsigned char CIPSTART[] = "AT+CIPSTART=\"TCP\",\"184.106.153.149\",80\r\n"; // Establish connection with ThingSpeak

volatile unsigned char CIPSEND[] = "AT+CIPSEND=104\r\n"; // Set send function to 104

volatile unsigned char CIPMUX[] = "AT+CIPMUX=0\r\n"; // Enable connection

volatile unsigned char SEND_DATA[] = "GET /update?key=IW250NQFTF4HL1KZ&field1="; // Get Write Key

volatile unsigned char RESET[] = "AT+RST\r\n"; // Get AT Firmware info

volatile unsigned char LINEBREAK[] = "\r\n"; // end of temperature transmission

volatile unsigned char CLOSE[] = "AT+CIPCLOSE\r\n";

//string for colors

volatile unsigned char RedStr[10];

volatile unsigned char GreenStr[10];

volatile unsigned char BlueStr[10];

uint16_t redVal, greenVal, blueVal;

int main(void){

i2c_init();

usart_init(115200);

init_APDS9960();

//Start up Esp

//Start AT communication

_delay_ms(10);

USART_putstring(AT);					//send AT to the USART

//connect to network

_delay_ms(10);

USART_putstring(RESET);		//reset ESP

_delay_ms(10);

USART_putstring(AT);			//confirm communication

_delay_ms(10);

USART_putstring(CWMODE);		//WiFi mode = 3

_delay_ms(10);

USART_putstring(CWJAP);		//Send wifi login

while(1){

//getValues();

_delay_ms(10);

USART_putstring(CIPMUX);		//Single connection point

_delay_ms(10);

USART_putstring(CIPSTART);	// Connect to ThingSpeak

_delay_ms(10);

USART_putstring(CIPSEND);	// Declare send length 50

_delay_ms(10);

getValues();

USART_putstring(SEND_DATA);	// Connect to proper key

USART_putstring(RedStr);		// Send adc data

USART_putstring("&field2=");

USART_putstring(GreenStr);		// Send adc data

USART_putstring("&field3=");

USART_putstring(BlueStr);		// Send adc data

_delay_ms(1000);

}

return 0;

}

void init_APDS9960(void){

uint8_t setup;

i2c_readReg(APDS9960_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS9960_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS9960_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS9960_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS9960_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS9960_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS9960_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void getValues(void){

uint8_t redVH, redVL;

uint8_t greenVH, greenVL;

uint8_t blueVH, blueVL;

unsigned char i;

char dummy[10];

// Read red value

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAH, &redVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_RDATAL, &redVL, 1);

// Read green value

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAH, &greenVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_GDATAL, &greenVL, 1);

// Read blue value

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAH, &blueVH, 1);

i2c_readReg(APDS9960_WRITE, APDS9960_BDATAL, &blueVL, 1);

redVal = (redVH << 8) | redVL;

greenVal = (greenVH << 8) | greenVL;

blueVal = (blueVH << 8) | blueVL;

// Set max threshold values

if (redVal > 255){

redVal = 255;

}

if (greenVal > 255){

greenVal = 255;

}

if (blueVal > 255){

blueVal = 255;

}

itoa(redVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

RedStr[i] = dummy[i]; //move converted ascii

}

itoa(greenVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

GreenStr[i] = dummy[i]; //move converted ascii

}

itoa(blueVal, dummy, 10); //convert char to ascii

for(i = 0 ; i < 10 ; i++){

BlueStr[i] = dummy[i]; //move converted ascii

}

}

void usart_init() {

UBRR0H = ((UBRR) >> 8);

UBRR0L = UBRR;

UCSR0A |= (1<< U2X0); // divisor baud = 8

UCSR0B |= (1 << TXEN0); // Enable transmission

UCSR0C |= (1 << UCSZ01) | (1 << UCSZ00); // 8 bits

}

void USART_putstring(volatile unsigned char *StringPtr)

{

while ((*StringPtr != '\0')){

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = *StringPtr;

StringPtr++;

}

}

- INCLUDE FILES

uart.h

/*

* uart.h

*

* Created: 12/8/2019 12:55:25 AM

*  Author: jreed

*/

#ifndef USART_RS232_H_FILE_H_				/* Define library H file if not defined */

#define USART_RS232_H_FILE_H_

#define F_CPU 16000000UL						/* Define CPU clock Frequency e.g. here its 8MHz */

#include <avr/io.h>							/* Include AVR std. library file */

#define BAUD_PRESCALE (((F_CPU / (BAUDRATE * 16UL))) - 1)	/* Define prescale value */

void USART_Init(unsigned long);				/* USART initialize function */

char USART_RxChar();						/* Data receiving function */

void USART_TxChar(char);					/* Data transmitting function */

void USART_SendString(char*);				/* Send string of USART data function */

#endif

uart.c

/*

* uart.c

*

* Created: 12/8/2019 12:54:27 AM

*  Author: jreed

*/

#include "uart.h"						/* Include USART header file */

void USART_Init(unsigned long BAUDRATE)				/* USART initialize function */

{

UCSR0B |= (1 << RXEN0) | (1 << TXEN0);				/* Enable USART transmitter and receiver */

UCSR0C |= (1 << UCSZ00) | (1 << UCSZ01);	/* Write USCRC for 8 bit data and 1 stop bit */

UBRR0L = BAUD_PRESCALE;							/* Load UBRRL with lower 8 bit of prescale value */

UBRR0H = (BAUD_PRESCALE >> 8);					/* Load UBRRH with upper 8 bit of prescale value */

}

char USART_RxChar()									/* Data receiving function */

{

while (!(UCSR0A & (1 << RXC0)));					/* Wait until new data receive */

return(UDR0);									/* Get and return received data */

}

void USART_TxChar(char data)						/* Data transmitting function */

{

UDR0 = data;										/* Write data to be transmitting in UDR */

while (!(UCSR0A & (1<<UDRE0)));					/* Wait until data transmit and buffer get empty */

}

void USART_SendString(char *str)					/* Send string of USART data function */

{

int i=0;

while (str[i]!=0)

{

USART_TxChar(str[i]);						/* Send each char of string till the NULL */

i++;

}

}

i2c_master.h

/*

* i2c_master.h

*

* Created: 12/8/2019 12:59:39 AM

*  Author: jreed

*/

#ifndef I2C_MASTER_H

#define I2C_MASTER_H

#define I2C_READ 0x01

#define I2C_WRITE 0x00

void i2c_init(void);

uint8_t i2c_start(uint8_t address);

uint8_t i2c_write(uint8_t data);

uint8_t i2c_read_ack(void);

uint8_t i2c_read_nack(void);

uint8_t i2c_transmit(uint8_t address, uint8_t* data, uint16_t length);

uint8_t i2c_receive(uint8_t address, uint8_t* data, uint16_t length);

uint8_t i2c_writeReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length);

uint8_t i2c_readReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length);

void i2c_stop(void);

#endif // I2C_MASTER_H

i2c_master.c

/*

* i2c_master.c

*

* Created: 12/8/2019 12:58:22 AM

*  Author: jreed

*/

#ifndef  F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/twi.h>

#include "i2c_master.h"

#define F_SCL 100000UL // SCL frequency

#define Prescaler 1

#define TWBR_val ((((F_CPU / F_SCL) / Prescaler) - 16 ) / 2)

#define TW_STATUS (TWSR0 & TW_STATUS_MASK)

void i2c_init(void)

{

TWBR0 = (uint8_t)TWBR_val;

}

uint8_t i2c_start(uint8_t address)

{

// reset TWI control register

TWCR0 = 0;

// transmit START condition

TWCR0 = (1<<TWINT) | (1<<TWSTA) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR0 & (1<<TWINT)) );

// check if the start condition was successfully transmitted

if((TWSR0 & 0xF8) != TW_START){ return 1; }

// load slave address into data register

TWDR0 = address;

// start transmission of address

TWCR0 = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR0 & (1<<TWINT)) );

// check if the device has acknowledged the READ / WRITE mode

uint8_t twst = TW_STATUS & 0xF8;

if ( (twst != TW_MT_SLA_ACK) && (twst != TW_MR_SLA_ACK) ) return 1;

return 0;

}

uint8_t i2c_write(uint8_t data)

{

// load data into data register

TWDR0 = data;

// start transmission of data

TWCR0 = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR0 & (1<<TWINT)) );

if( (TWSR0 & 0xF8) != TW_MT_DATA_ACK ){ return 1; }

return 0;

}

uint8_t i2c_read_ack(void)

{

// start TWI module and acknowledge data after reception

TWCR0 = (1<<TWINT) | (1<<TWEN) | (1<<TWEA);

// wait for end of transmission

while( !(TWCR0 & (1<<TWINT)) );

// return received data from TWDR

return TWDR0;

}

uint8_t i2c_read_nack(void)

{

// start receiving without acknowledging reception

TWCR0 = (1<<TWINT) | (1<<TWEN);

// wait for end of transmission

while( !(TWCR0 & (1<<TWINT)) );

// return received data from TWDR

return TWDR0;

}

uint8_t i2c_transmit(uint8_t address, uint8_t* data, uint16_t length)

{

if (i2c_start(address | I2C_WRITE)) return 1;

for (uint16_t i = 0; i < length; i++)

{

if (i2c_write(data[i])) return 1;

}

i2c_stop();

return 0;

}

uint8_t i2c_receive(uint8_t address, uint8_t* data, uint16_t length)

{

if (i2c_start(address | I2C_READ)) return 1;

for (uint16_t i = 0; i < (length-1); i++)

{

data[i] = i2c_read_ack();

}

data[(length-1)] = i2c_read_nack();

i2c_stop();

return 0;

}

uint8_t i2c_writeReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length)

{

if (i2c_start(devaddr | 0x00)) return 1;

i2c_write(regaddr);

for (uint16_t i = 0; i < length; i++)

{

if (i2c_write(data[i])) return 1;

}

i2c_stop();

return 0;

}

uint8_t i2c_readReg(uint8_t devaddr, uint8_t regaddr, uint8_t* data, uint16_t length)

{

if (i2c_start(devaddr)) return 1;

i2c_write(regaddr);

if (i2c_start(devaddr | 0x01)) return 1;

for (uint16_t i = 0; i < (length-1); i++)

{

data[i] = i2c_read_ack();

}

data[(length-1)] = i2c_read_nack();

i2c_stop();

return 0;

}

void i2c_stop(void)

{

// transmit STOP condition

TWCR0 = (1<<TWINT) | (1<<TWEN) | (1<<TWSTO);

}

APDS9960_def.h

/*

* APDS9960_def.h

*

* Created: 12/8/2019 2:04:11 AM

*  Author: jreed

*/

#ifndef SparkFun_APDS9960_H

#define SparkFun_APDS9960_H

/* APDS-9960 I2C address */

#define APDS9960_I2C_ADDR       0x39

/* Gesture parameters */

#define GESTURE_THRESHOLD_OUT   10

#define GESTURE_SENSITIVITY_1   50

#define GESTURE_SENSITIVITY_2   20

/* Error code for returned values */

#define ERROR                   0xFF

/* Acceptable device IDs */

#define APDS9960_ID_1           0xAB

#define APDS9960_ID_2           0x9C

/* Misc parameters */

#define FIFO_PAUSE_TIME         30      // Wait period (ms) between FIFO reads

/* APDS-9960 register addresses */

#define APDS9960_ENABLE         0x80

#define APDS9960_ATIME          0x81

#define APDS9960_WTIME          0x83

#define APDS9960_AILTL          0x84

#define APDS9960_AILTH          0x85

#define APDS9960_AIHTL          0x86

#define APDS9960_AIHTH          0x87

#define APDS9960_PILT           0x89

#define APDS9960_PIHT           0x8B

#define APDS9960_PERS           0x8C

#define APDS9960_CONFIG1        0x8D

#define APDS9960_PPULSE         0x8E

#define APDS9960_CONTROL        0x8F

#define APDS9960_CONFIG2        0x90

#define APDS9960_ID             0x92

#define APDS9960_STATUS         0x93

#define APDS9960_CDATAL         0x94

#define APDS9960_CDATAH         0x95

#define APDS9960_RDATAL         0x96

#define APDS9960_RDATAH         0x97

#define APDS9960_GDATAL         0x98

#define APDS9960_GDATAH         0x99

#define APDS9960_BDATAL         0x9A

#define APDS9960_BDATAH         0x9B

#define APDS9960_PDATA          0x9C

#define APDS9960_POFFSET_UR     0x9D

#define APDS9960_POFFSET_DL     0x9E

#define APDS9960_CONFIG3        0x9F

#define APDS9960_GPENTH         0xA0

#define APDS9960_GEXTH          0xA1

#define APDS9960_GCONF1         0xA2

#define APDS9960_GCONF2         0xA3

#define APDS9960_GOFFSET_U      0xA4

#define APDS9960_GOFFSET_D      0xA5

#define APDS9960_GOFFSET_L      0xA7

#define APDS9960_GOFFSET_R      0xA9

#define APDS9960_GPULSE         0xA6

#define APDS9960_GCONF3         0xAA

#define APDS9960_GCONF4         0xAB

#define APDS9960_GFLVL          0xAE

#define APDS9960_GSTATUS        0xAF

#define APDS9960_IFORCE         0xE4

#define APDS9960_PICLEAR        0xE5

#define APDS9960_CICLEAR        0xE6

#define APDS9960_AICLEAR        0xE7

#define APDS9960_GFIFO_U        0xFC

#define APDS9960_GFIFO_D        0xFD

#define APDS9960_GFIFO_L        0xFE

#define APDS9960_GFIFO_R        0xFF

/* Bit fields */

#define APDS9960_PON            0b00000001

#define APDS9960_AEN            0b00000010

#define APDS9960_PEN            0b00000100

#define APDS9960_WEN            0b00001000

#define APSD9960_AIEN           0b00010000

#define APDS9960_PIEN           0b00100000

#define APDS9960_GEN            0b01000000

#define APDS9960_GVALID         0b00000001

/* On/Off definitions */

#define OFF                     0

#define ON                      1

/* Acceptable parameters for setMode */

#define POWER                   0

#define AMBIENT_LIGHT           1

#define PROXIMITY               2

#define WAIT                    3

#define AMBIENT_LIGHT_INT       4

#define PROXIMITY_INT           5

#define GESTURE                 6

#define ALL                     7

/* LED Drive values */

#define LED_DRIVE_100MA         0

#define LED_DRIVE_50MA          1

#define LED_DRIVE_25MA          2

#define LED_DRIVE_12_5MA        3

/* Proximity Gain (PGAIN) values */

#define PGAIN_1X                0

#define PGAIN_2X                1

#define PGAIN_4X                2

#define PGAIN_8X                3

/* ALS Gain (AGAIN) values */

#define AGAIN_1X                0

#define AGAIN_4X                1

#define AGAIN_16X               2

#define AGAIN_64X               3

/* Gesture Gain (GGAIN) values */

#define GGAIN_1X                0

#define GGAIN_2X                1

#define GGAIN_4X                2

#define GGAIN_8X                3

/* LED Boost values */

#define LED_BOOST_100           0

#define LED_BOOST_150           1

#define LED_BOOST_200           2

#define LED_BOOST_300           3

/* Gesture wait time values */

#define GWTIME_0MS              0

#define GWTIME_2_8MS            1

#define GWTIME_5_6MS            2

#define GWTIME_8_4MS            3

#define GWTIME_14_0MS           4

#define GWTIME_22_4MS           5

#define GWTIME_30_8MS           6

#define GWTIME_39_2MS           7

/* Default values */

#define DEFAULT_ATIME           219     // 103ms

#define DEFAULT_WTIME           246     // 27ms

#define DEFAULT_PROX_PPULSE     0x87    // 16us, 8 pulses

#define DEFAULT_GESTURE_PPULSE  0x89    // 16us, 10 pulses

#define DEFAULT_POFFSET_UR      0       // 0 offset

#define DEFAULT_POFFSET_DL      0       // 0 offset

#define DEFAULT_CONFIG1         0x60    // No 12x wait (WTIME) factor

#define DEFAULT_LDRIVE          LED_DRIVE_100MA

#define DEFAULT_PGAIN           PGAIN_4X

#define DEFAULT_AGAIN           AGAIN_4X

#define DEFAULT_PILT            0       // Low proximity threshold

#define DEFAULT_PIHT            50      // High proximity threshold

#define DEFAULT_AILT            0xFFFF  // Force interrupt for calibration

#define DEFAULT_AIHT            0

#define DEFAULT_PERS            0x11    // 2 consecutive prox or ALS for int.

#define DEFAULT_CONFIG2         0x01    // No saturation interrupts or LED boost

#define DEFAULT_CONFIG3         0       // Enable all photodiodes, no SAI

#define DEFAULT_GPENTH          40      // Threshold for entering gesture mode

#define DEFAULT_GEXTH           30      // Threshold for exiting gesture mode

#define DEFAULT_GCONF1          0x40    // 4 gesture events for int., 1 for exit

#define DEFAULT_GGAIN           GGAIN_4X

#define DEFAULT_GLDRIVE         LED_DRIVE_100MA

#define DEFAULT_GWTIME          GWTIME_2_8MS

#define DEFAULT_GOFFSET         0       // No offset scaling for gesture mode

#define DEFAULT_GPULSE          0xC9    // 32us, 10 pulses

#define DEFAULT_GCONF3          0       // All photodiodes active during gesture

#define DEFAULT_GIEN            0       // Disable gesture interrupts

#endif

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/5QeY6dw3u6M

- GITHUB LINK OF THIS DA

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT