CPE301 – FALL 2019

# MIDTERM 1

Student Name: Jacob Patrick Reed

Student #: 1008448895

Student Email: reedj35@unlv.nevada.edu

Primary Github address: https://github.com/reedjacobp

Directory: https://github.com/reedjacobp/submission_da

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

- Atmega328PB Xplained Mini

- LM35

- ESP01

- Multifunction Shield

FTDI Vcc  ESP01 3.3V & EN

ATmega328PB Xplained Mini GND  ESP01 GND

Multifunction Shield APC220 Bluetooth Voice Recognition Module TX (Pin 4)  ESP01 RX

Multifunction Shield APC220 Bluetooth Voice Recognition Module TX (Pin 4)  ESP01 RX

Multifunction Shield PC4 GND (Pin 1)  LM35 GND (Pin 3)

Multifunction Shield PC4 Data (Pin 2)  LM35 Vout (Pin 2)

Multifunction Shield PC4 Vcc +5V (Pin 3)  LM35 Vs (Pin 1)

- DEVELOPED CODE

/*

* Midterm1.c

*

* Created: 10/25/2019 2:07:45 PM

* Author : jreed

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#include <util/delay.h>

#include <stdio.h>

#include <stdint.h>

#include <stdlib.h>

#include <math.h>

#define BAUDRATE 115200

#define BAUD_PRESCALLER (int)round(((((double)F_CPU / ((double)BAUDRATE * 8.0))) - 1.0))

void USART_init(void); //function to initialize USART

void USART_send(unsigned char data); //function to send through USART

void USART_putstring(char* StringPtr); //goes through each character in a string to send through USART

void ADC_init(void); //used to initialize analog to digital conversion

uint8_t OVF_COUNT = 0; //initialize the overflow count for interrupt

uint8_t OVF_LIMIT = 250; //set the limit the count can reach to set 1 sec delay

int main(void)

{

USART_init(); //initialize USART

ADC_init(); //initialize conversion

USART_putstring("AT+CWMODE=1\r\n"); //sets ESP01 in station mode, all of the \r\n seen throught the code is to "press enter/return" and create a new line

_delay_ms(10); //the delays seen throughout the code is so that when strings are sent, there's enough time to process

USART_putstring("AT+CWJAP=\"SBG6700AC-8AD50\",\"98aa7a769e\"\r\n"); //set wifi and password here

_delay_ms(10);

USART_putstring("AT+CIPMUX=0\r\n"); //sets ESP01 to have a single connection

_delay_ms(10);

TCCR0A = 0x00; //normal operation

TCCR0B |= (1 << CS02); //set prescalar to 256

TCNT0 = 16; //TOP = 256-250 = 16

TIMSK0 |= (1 << TOIE0);

sei(); //enable interrupt

while (1)

{

}

}

void USART_init(void)

{

UCSR0A = (1 << U2X0); //double USART transmission speed

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8); //set the baud rate register

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0); //enable transmit and receive

UCSR0C = (3 << UCSZ00); //sets character size to 8-bits

}

void USART_putstring(char* StringPtr)

{

while(*StringPtr != 0x00)

{

USART_send(*StringPtr);

StringPtr++;

}

}

void USART_send(unsigned char data) {			// Function to transmit ASCII value into UDR0

while (!(UCSR0A & (1 << UDRE0)));			// Keep Checking until  UDRE0 data register 'High' to break loop

UDR0 = data;								// Store unsigned char serial data into UDR0

}

void ADC_init(void)

{

ADMUX = (0<<REFS1)|	// reference selection bits

(1<<REFS0)|	// AVcc - external cap at AREF

(0<<ADLAR)|	// ADC Left Adjust Result

(1<<MUX2)|	// Analog Channel Selection Bits

(0<<MUX1)|	// ADC4 (PC4)

(0<<MUX0);

ADCSRA = (1<<ADEN)|	// ADC Enable

(0<<ADSC)|	// ADC Start Conversion

(0<<ADATE)|	// ADC Auto Trigger Enable

(0<<ADIF)|	// ADC Interrupt Flag

(0<<ADIE)|	// ADC Interrupt Enable

(1<<ADPS2)|	// ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

ISR (TIMER0_OVF_vect)

{

OVF_COUNT++; //increment the overflow counter

if (OVF_COUNT == OVF_LIMIT) //check to see if the limit was reached

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

int a = ADCL; //a is temperature

a = a | (ADCH << 8);

a = (a/1024.0) * 5000/10;

a = (a*2)+32; //equation to convert celsius to fahrenheit. cannot use 9/5, 2 must be used

a = a % 100;

USART_putstring("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n"); //connect to thingspeak

_delay_ms(10);

USART_putstring("AT+CIPSEND=51\r\n"); //start sending data in transparent transmission mode

_delay_ms(10);

USART_putstring("GET /update?key=YLVIWUJ8MIHKPKCR&field1="); //prepare to send data using "Write" API Key

_delay_ms(10);

USART_send((a/10)+'0');

_delay_ms(10);

a = a % 10;

USART_send((a)+'0');

_delay_ms(10);

USART_putstring(" ");

_delay_ms(10);

USART_putstring("\r\n");

_delay_ms(10);

OVF_COUNT = 0; //reset overflow counter

}

TCNT0 = 16; //reset TOP

}

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/6JFHt2f-pCQ

- GITHUB LINK OF THIS MIDTERM

https://github.com/reedjacobp/submission_da/tree/master/Midterms/Midterm%201

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT